| # Arithmetic Logic Unit |  |  |
| --- | --- | --- |
|  | ## Date: 29/11/2025 ## Processor Architecture (EECG241-17) |

Our Team

# Code; # Section; # Name; # ID. # 91240733; # 4; # مريم احمد محمد علي رضا; # 1. # 91240202; # 1; # باسل شريف حميد مصطفى; # 2.

| Instructor: Eng. Ahmed Atef |  |
| --- | --- |

Codes are Uploaded in our Github Repositary

Table of contents:
      1- Introduction

2- ALU Architecture overview

1. Arithmetic Unit
	 2. Logic Unit
	 3. Shift Unit

3- Verilog Code

1. Arithmetic Unit
	 2. Logic Unit
	 3. Shift Unit

4. ALU Unit
	 5. ALU_TB Unit

4- Testbench

5- Do File

1. Introduction

In this project, we designed a 16-bit combinational ALU that implements the required 20 functions across three major blocks:

- Arithmetic Unit

- Logic Unit

- Shift/Rotate Unit

Our Project ALU Codes are uploaded to the Github Repositary

The main objective was to implement all operations and generate the 6 status flags (C, Z, N, V, P, A) as required in the assignment brief (page 1–3 of the PDF)

The main Design of the ALU was from Dr. Ibrahim Qamar Slides in Course Processor Architecture.

2. ALU Architecture Overview

The ALU uses the upper bits of F[4:3] to select a block:

00 → Arithmetic

01 → Logic

10 → Shift/Rotate

2.1 Arithmetic Unit

Implements: INC, DEC, ADD, ADD+CIN, SUB, SUB−CIN.
Generates flags: C, Z, N, V, P, Af.

2.2 Logic Unit

Implements: AND, OR, XOR, NOT.
Generates Z, N, P.

2.3 Shift Unit

Implements: SHL, SHR, SAL, SAR, ROL, ROR, RCL, RCR.
Carry = shifted/rotated-out bit.

3. Verilog Code

3.1 Arithmetic Unit

tmodule Arithmetic

#(parameter Width=16)

(

input       [Width-1:0] A,B,

input       Cin,

input       [2:0] F,

output reg  [Width-1:0] Out,

output reg  Cout,

output reg  [5:0] Status,

output reg  C,Z,N,V,P,Af

);

reg [Width:0] temp;

reg overflow;

always @(*)

begin

case(F)

3'b001:					//INC

begin

temp=A+1;

Out=temp[Width-1:0];

Cout=temp[Width];

if(A[Width-1]^Out[Width-1])

overflow=1'b1;

else

overflow=1'b0;

end

////////////////////////////////////////////////////////////

3'b011:					//DEC

begin

temp=A-1;

Out=temp[Width-1:0];

Cout=temp[Width];

if(A[Width-1]^Out[Width-1])

overflow=1'b1;

else

overflow=1'b0;

end

////////////////////////////////////////////////////////////

3'b100:					//ADD

begin

temp=A+B;

Out=temp[Width-1:0];

Cout=temp[Width];

if( (A[Width-1] & B[Width-1]& ~Out[Width-1]) | ( ~A[Width-1] & ~B[Width-1]& Out[Width-1]) )

overflow=1'b1;

else

overflow=1'b0;

end

////////////////////////////////////////////////////////////

3'b101:					//ADD_CARRY

begin

temp=A+B+Cin;

Out=temp[Width-1:0];

Cout=temp[Width];

if( (A[Width-1] & B[Width-1]& ~Out[Width-1]) | ( ~A[Width-1] & ~B[Width-1]& Out[Width-1]) )

overflow=1'b1;

else

overflow=1'b0;

end

////////////////////////////////////////////////////////////

3'b110:					//SUB

begin

temp=A-B;

Out=temp[Width-1:0];

Cout=temp[Width];

if( (A[Width-1] & ~B[Width-1]& ~Out[Width-1]) | ( ~A[Width-1] & B[Width-1]& Out[Width-1]) )

overflow=1'b1;

else

overflow=1'b0;

end

////////////////////////////////////////////////////////////

3'b111:					//SUB_BORROW

begin

temp=A-B-Cin;

Out=temp[Width-1:0];

Cout=temp[Width];

if( (A[Width-1] & ~B[Width-1]& ~Out[Width-1]) | ( ~A[Width-1] & B[Width-1]& Out[Width-1]) )

overflow=1'b1;

else

overflow=1'b0;

end

////////////////////////////////////////////////////////////

default:

begin

Out=0;

Cout=0;

overflow=0;

end

////////////////////////////////////////////////////////////

endcase

////////////////////////////////////////////////////////////

C  = (F[2] | F[0]) ? Cout                          : 1'b0;                     		// Carry Flag

Z  = (F[2] | F[0]) ? (Out=={Width{1'b0}})          : 1'b0;                     		// Zero Flag

N  = (F[2] | F[0]) ? Out[Width-1]                  : 1'b0;                     		// Negative Flag

V  = (F[2] | F[0]) ? overflow                      : 1'b0;                     		// Overflow Flag

P  = (F[2] | F[0]) ? ~^Out                         : 1'b0;                     		// Parity Flag

Af = (F[2] | F[0]) ? ((A[3:0]+B[3:0]+Cin)>4'hF)    : 1'b0;                     		// Auxiliary Flag

// The if condition here is just to make the Flags = 0 when the two not used OP Codes in the Project Details

Status={C,Z,N,V,P,Af}; // Concatenate

end

endmodule

3.2 Logic Unit

module Logic

#(parameter Width = 16)

(

input      [Width-1:0] A,B,

input      [2:0] F,

output reg Z,N,P,

output reg [Width-1:0] Out

);

always@(*) begin

case(F)

3'b000:					//AND

begin

Out = A & B;

Z = ~|Out;		//Zero Flag

N = Out[Width-1];	//Negative Flag

P = ~^Out;		//Parity Flag

end

////////////////////////////////////////////////////////////

3'b001:					//OR

begin

Out = A | B;

Z = ~|Out;		//Zero Flag

N = Out[Width-1];	//Negative Flag

P = ~^Out;		//Parity Flag

end

////////////////////////////////////////////////////////////

3'b010:					//XOR

begin

Out = A ^ B;

Z = ~|Out;		//Zero Flag

N = Out[Width-1];	//Negative Flag

P = ~^Out;		//Parity Flag

end

////////////////////////////////////////////////////////////

3'b011:					//NOT

begin

Out = ~A;

Z = ~|Out;		//Zero Flag

N = Out[Width-1];	//Negative Flag

P = ~^Out;		//Parity Flag

end

////////////////////////////////////////////////////////////

default:

begin

Out = 1'b0;

Z = 1'b0;		//Zero Flag

N = 1'b0;		//Negative Flag

P = 1'b0;		//Parity Flag

end

////////////////////////////////////////////////////////////

endcase

end

endmodule

3.3 Shift Unit

module Shift

#(parameter Width=16)

(

input  	   [Width-1:0] A,

input      [2:0] F,

output reg C,Z,N,P,

output reg [Width-1:0] Out

);

reg Cshift;

always @(*)

begin

case (F)

3'b000:					//SHL

{C,Out} = {A,1'b0};

////////////////////////////////////////////////////////////

3'b001:					//SHR

{Out,C} = {1'b0,A};

////////////////////////////////////////////////////////////

3'b010:					//SAL

{C,Out} = {A,1'b0};

////////////////////////////////////////////////////////////

3'b011:					//SAR

{Out,C} = {A[Width-1],A};

////////////////////////////////////////////////////////////

3'b100:					//ROL

{C,Out} = {A,A[Width-1]};

////////////////////////////////////////////////////////////

3'b101:					//ROR

{Out,C} = {A[0],A};

////////////////////////////////////////////////////////////

3'b110:					//RCL

begin

Cshift = C;

{C,Out} = {A,Cshift};

end

////////////////////////////////////////////////////////////

3'b111:					//RCR

begin

Cshift = C;

{Out,C} = {Cshift,A};

end

////////////////////////////////////////////////////////////

default:

begin

Out = {Width{1'b0}};

C   = 1'b0;

Z 	= 1'b0;

N 	= 1'b0;

P 	= 1'b0;

end

////////////////////////////////////////////////////////////

endcase

Z = ~|Out;		//Zero Flag

N = Out[Width-1];	//Negative Flag

P = ~^Out;		//Parity Flag

end

endmodule

3.4 ALU Unit

module ALU

#(parameter Width=16)

(

input       [Width-1:0] A,B,

input       Cin,

input       [4:0] F,                 // F[4:3] -> block select, lower bits -> sub-op

output reg  [Width-1:0] Out,

output reg  [5:0] Status

);

// Registers for the Flags Used

reg        C,Z,N,V,P,Af,Cout;

// Arithmetic Wires

wire [Width-1:0] arith_out;

wire             arith_C,arith_Z,arith_N,arith_V,arith_P,arith_Af;

// Logic Wires

wire [Width-1:0] logic_out;

wire             logic_Z,logic_N,logic_P;

// Shift Wires

wire [Width-1:0] shift_out;

wire             shift_C,shift_Z,shift_N,shift_P;

// ================== Instantiations ==================

// Arithmetic block (F[2:0] from ALU F)

Arithmetic#(.Width(Width)) U_ARITH (

.A(A),

.B(B),

.Cin(Cin),

.F(F[2:0]),

.Out(arith_out),

.Cout(),          // not used

.Status(),        // not used

.C(arith_C),

.Z(arith_Z),

.N(arith_N),

.V(arith_V),

.P(arith_P),

.Af(arith_Af)

);

// Logic block (F[1:0] from ALU F)

Logic#(.Width(Width)) U_LOGIC (

.A(A),

.B(B),

.F(F[2:0]),

.Z(logic_Z),

.N(logic_N),

.P(logic_P),

.Out(logic_out)

);

// Shift block (F[2:0] from ALU F)

Shift#(.Width(Width)) U_SHIFT (

.A(A),

.F(F[2:0]),

.Out(shift_out),

.C(shift_C),

.Z(shift_Z),

.N(shift_N),

.P(shift_P)

);

// ================== ALU Mux Logic ==================

always@(*) begin

case (F[4:3])

2'b00:

begin

// Arithmetic operations

Out = arith_out;

C   = arith_C;

Z   = arith_Z;

N   = arith_N;

V   = arith_V;

P   = arith_P;

Af  = arith_Af;

end

////////////////////////////////////////////////////////////

2'b01:

begin

// Logic operations

Out = logic_out;

C   = 1'b0;

Z   = logic_Z;

N   = logic_N;

V   = 1'b0;

P   = logic_P;

Af  = 1'b0;

end

////////////////////////////////////////////////////////////

2'b10:

begin

// Shift operations

Out = shift_out;

C   = shift_C;

Z   = shift_Z;

N   = shift_N;

V   = 1'b0;

P   = shift_P;

Af  = 1'b0;

end

////////////////////////////////////////////////////////////

default:

begin

// Unused opcodes ------> everything zero

Out = {Width{1'b0}};

C   = 1'b0;

Z   = 1'b0;

N   = 1'b0;

V   = 1'b0;

P   = 1'b0;

Af  = 1'b0;

end

////////////////////////////////////////////////////////////

endcase

Cout   = C;

Status = {C,Z,N,V,P,Af};

end

endmodule

3.5 ALU_TB Unit

`timescale 1ns/1ps

module ALU_tb

#(parameter Width = 16);

reg [Width-1:0] A,B;

reg [4:0] F;

reg Cin;

wire[Width-1:0] Out;

wire[5:0] Status;

// Instanstaite of ALI Block

ALU#(.Width(Width)) U_ALU (

.A(A),

.B(B),

.Cin(Cin),

.F(F),

.Out(Out),

.Status(Status)

);

initial

begin

$display(" time    |        A        |        B        | Cin |   F   |       Out       | Status");

$display("----------------------------------------------------------------------------------------");

F   = 5'b00000;

repeat(100)

begin

A   = $random;

B   = $random;

Cin = 1;        //$random;

#1

$display("%5t  | %b  %b   %b  %b  | %b   %b",

$time, A, B, Cin, F, Out, Status);

F = F + 1;

#10;

end

end

endmodule

4. Testbench

#  time  |                       A                       |                      B                   | Cin |    F    |                Out                       |     Status

# ------------------------------------------------------------------------------------------------------------------------------------------------

# 10000    | 0011010100100100  13604  0101111010000001  24193   1   00000  | 0000000000000000      0          000000

# 20000    | 1101011000001001  54793  0101011001100011  22115   1   00001  | 1101011000001010  54794      001000

# 30000    | 0111101100001101  31501  1001100110001101  39309   1   00010  | 0000000000000000      0          000000

# 40000    | 1000010001100101  33893  0101001000010010  21010   1   00011  | 1000010001100100  33892      001000

# 50000    | 1110001100000001  58113  1100110100001101  52493   1   00100  | 1011000000001110  45070      101010

# 60000    | 1111000101110110  61814  1100110100111101  52541   1   00101  | 1011111010110100  48820      101010

# 70000    | 0101011111101101  22509  1111011110001100  63372   1   00110  | 0110000001100001  24673      100000

# 80000    | 1110100111111001  59897  0010010011000110   9414    1   00111  | 1100010100110010  50482      001000

# 90000    | 1000010011000101  33989  1101001010101010  53930   1   01000  | 1000000010000000  32896      001010

# 100000  | 1111011111100101  63461  0111001001110111  29303   1   01001  | 1111011111110111  63479      001010

# 110000  | 1101011000010010  54802  1101101110001111  56207   1   01010  | 0000110110011101   3485       000010

# 120000  | 0110100111110010  27122  1001011011001110  38606   1   01011  | 1001011000001101  38413      001000

# 130000  | 0111101011101000  31464  0100111011000101  20165   1   01100  | 0000000000000000      0          000000

# 140000  | 0100100101011100  18780  0010100010111101  10429   1   01101  | 0000000000000000      0          000000

# 150000  | 0101100000101101  22573  0010011001100101   9829    1   01110  | 0000000000000000      0          000000

# 160000  | 0110001001100011  25187  1000011100001010  34570   1   01111  | 0000000000000000      0          000000

# 170000  | 0010001010000000   8832  0010000100100000   8480     1   10000  | 0100010100000000  17664      000000

# 180000  | 0100010110101010  17834  1100110010011101  52381   1   10001  | 0010001011010101   8917       000000

# 190000  | 0011111010010110  16022  1011100000010011  47123   1   10010  | 0111110100101100  32044      000000

# 200000  | 0011100000001101  14349  1101011001010011  54867   1   10011  | 0001110000000110   7174       100000

# 210000  | 1101110101101011  56683  0010101011010101  10965   1   10100  | 1011101011010111  47831      101000

# 220000  | 0100101000000010  18946  0011111010101110  16046   1   10101  | 0010010100000001   9473       000010

# 230000  | 1110100100011101  59677  0111001011001111  29391   1   10110  | 1101001000111010  53818      101010

# 240000  | 0100100100100011  18723  0110010100001010  25866   1   10111  | 1010010010010001  42129      101010

# 250000  | 0000101011001010   2762  0100110000111100  19516    1   11000  | 0000000000000000      0          000000

# 260000  | 1011110111110010  48626  0110000110001010  24970   1   11001  | 0000000000000000      0          000000

# 270000  | 1011001101000001  45889  0011010011011000  13528   1   11010  | 0000000000000000      0          000000

# 280000  | 1111001101111000  62328  0001001010001001   4745    1   11011  | 0000000000000000      0          000000

# 290000  | 0000110111101011   3563  0110010110110110  26038    1   11100  | 0000000000000000      0          000000

# 300000  | 1111100111000110  63942  0001001110101110   5038    1   11101  | 0000000000000000      0          000000

# 310000  | 0000001010111100    700  1101110100101010  56618     1   11110  | 0000000000000000      0          000000

# 320000  | 1001101000001011  39435  1011111001110001  48753   1   11111  | 0000000000000000      0          000000

# 330000  | 0100000110000101  16773  0101010101001111  21839   1   00000  | 0000000000000000      0          000000

# 340000  | 0110000000111011  24635  0011001100111010  13114   1   00001  | 0110000000111100  24636      000010

# 350000  | 0011001001111110  12926  0100101100010101  19221   1   00010  | 0000000000000000      0          000000

5. Do file

vlib work

vlog ALU.v ALU_TB.v

vsim -voptargs=+acc work.ALU_tb

add wave -r /*

run -all

To run the simulation the Transcript we type :

Do run.txt